EEEN 464 – DIGITAL COMMUNICATION

LINE CODING - STUDY GUIDE/REVISION

1. INTRODUCTION TO LINE CODING

- Definition: Line coding converts digital data (binary bits) into electrical/optical signals for transmission over physical media (e.g., cables, fibre optics).

- Purpose:

  - Enable synchronization between transmitter/receiver.

  - Reduce transmission errors.

  - Minimize signal distortion (e.g., DC component).

- Key Concepts:

  - Symbol: A voltage/current level representing one or more bits.

  - Pulse: A signal waveform (e.g., positive/negative voltage, zero).

2. KEY CHARACTERISTICS OF LINE CODES

| CHARACTERISTIC | DESCRIPTION |
| --- | --- |
| Polarity | Unipolar (one voltage level), Polar (positive/negative levels), Bipolar (three levels). |
| Voltage Levels | Number of signal states (e.g., 2 levels for binary). |
| Bit Rate vs. Baud Rate | Bit rate = data speed; Baud rate = signal changes per second. |
| DC Component | Presence of low-frequency signal drift; undesirable in transformers/capacitors. |
| Self-Clocking | Built-in timing information for synchronization. |
| Bandwidth | Minimum frequency range needed for transmission. |
| Noise Immunity | Resistance to signal corruption. |

3. COMMON LINE CODING SCHEMES

- UNIPOLAR NRZ (NON-RETURN-TO-ZERO)

Encoding:

  - 0 = 0V

  - 1 = +V

- Pros: Simple.

- Cons: DC component; no synchronization for long 0/1 sequences.

- POLAR SCHEMES

- NRZ-L (NRZ-Level):

  - 0 = +V, 1 = –V.

- NRZ-I (NRZ-Invert):

  - 0 = no change, 1 = voltage inversion.

- RZ (Return-to-Zero):

  - Each bit ends at zero (half-duration pulse).

  - E.g., 1 = +V → 0V; 0 = –V → 0V.

- Pros: Reduced DC component (RZ).

- Cons: Higher bandwidth (RZ), no clocking (NRZ).

- BIPHASE SCHEMES

- Manchester:

  - 0 = Low→High transition; 1 = High→Low transition (mid-bit).

  - Pros: Self-clocking; no DC component.

- Differential Manchester:

  - Transition at mid-bit = clocking.

  - 0 = additional transition at start; 1 = no transition.

  - Pros: Robust noise immunity.

- BIPOLAR SCHEMES

- AMI (Alternate Mark Inversion):

  - 0 = 0V; 1 alternates between +V/–V.

- HDB3/B8ZS:

  - AMI variants that prevent long zero sequences.

- Pros: No DC component; error detection.

4. SELECTION CRITERIA

- Bandwidth Efficiency: RZ uses 2x NRZ bandwidth.

- Synchronization: Manchester/AMI support self-clocking.

- Complexity: Unipolar is simplest; Bipolar needs advanced encoders.

- Error Detection: AMI detects errors via polarity violations.

- DC Balance: Bipolar/Manchester avoid DC drift.

5. ADVANTAGES/DISADVANTAGES

| Scheme | Advantages | Disadvantages |
| --- | --- | --- |
| Unipolar NRZ | Simple implementation. | DC component; no synchronization. |
| Polar NRZ | Low bandwidth. | Baseline wander; no clocking. |
| RZ | Self-clocking; no DC drift. | Double bandwidth. |
| Manchester | Self-clocking; no DC; error tolerance. | High bandwidth. |
| AMI | No DC; error detection; low bandwidth. | Complex; loses sync for long zeros. |

6. PRACTICE PROBLEMS

- Encode 1011001 using:

  - Unipolar NRZ

  - Manchester

  - AMI
Solutions:

  - Unipolar: +V 0V +V +V 0V 0V +V

  - Manchester: H→L, L→H, H→L, H→L, L→H, L→H, H→L (mid-bit transitions).

  - AMI: +V 0V –V +V 0V 0V –V (alternating 1s).

- Which scheme is ideal for Ethernet?

  - Answer: Manchester (used in classic Ethernet).

- Why does AMI avoid DC drift?

  - Answer: Equal positive/negative pulses average to zero DC.