EEEN 464 – DIGITAL COMMUNICATION

LINE CODING – TEST YOUR KNOWLEDGE

SECTION 1: FUNDAMENTALS

- What is line coding?

  - a) Converting analogue signals to digital

  - b) Mapping binary data to voltage/light pulses for transmission

  - c) Compressing data for efficient storage

  - d) Encrypting data for security
*Explanation: Line coding transforms binary bits (0s/1s) into physical signals (e.g., voltage levels) to send over wires or fibre optics.*

- Why is synchronization important in line coding?

  - a) To reduce bandwidth usage

  - b) To ensure the receiver correctly samples bits

  - c) To increase transmission voltage

  - d) To encrypt data
Explanation: Without synchronization, the receiver can’t distinguish where one bit ends and another begins, causing errors.

- The absence of a DC component in a line code helps:

  - a) Increase transmission speed

  - b) Prevent signal distortion in transformers/capacitors

  - c) Simplify encoding logic

  - d) Reduce error rates
Explanation: DC components cause signal drift and are blocked by components like transformers.

- True or False: Bit rate and baud rate are always equal.

  - False
*Explanation: Bit rate = data speed (bps); Baud rate = signal changes/sec. They differ in multi-level schemes (e.g., 2 bits/symbol).*

- A "self-clocking" line code:

  - a) Uses an external clock signal

  - b) Embeds timing information in the signal

  - c) Requires GPS synchronization

  - d) Slows down transmission
Explanation: Self-clocking codes (e.g., Manchester) include regular signal transitions to help the receiver sync.

SECTION 2: LINE CODING SCHEMES

- In Unipolar NRZ, a binary '1' is represented by:

  - a) 0V

  - b) +V

  - c) –V

  - d) Alternating +V/–V
*Explanation: Unipolar NRZ uses 0V for '0' and +V for '1'.*

- Which scheme suffers from "baseline wandering"?

  - a) Manchester

  - b) NRZ-L

  - c) AMI

  - d) RZ
Explanation: Long sequences of identical bits in NRZ cause voltage drift (baseline wandering), making synchronization hard.

- In Manchester encoding, a binary '1' is:

  - a) High-to-Low transition at bit start

  - b) High-to-Low transition at mid-bit

  - c) Low-to-High transition at bit start

  - d) Steady high voltage
*Explanation: Manchester uses mid-bit transitions: High→Low for '1', Low→High for '0'.*

- Differential Manchester encoding differs from standard Manchester by:

  - a) Using three voltage levels

  - b) Encoding data at the start-of-bit transition

  - c) Eliminating all transitions

  - d) Doubling the bit rate
Explanation: Data is in the presence/absence of a transition at the bit start (not mid-bit).

- Bipolar AMI encodes a binary '0' as:

  - a) Alternating +V/–V

  - b) 0V

  - c) +V

  - d) –V
*Explanation: AMI uses 0V for '0' and alternates between +V/–V for '1's.*

- HDB3 and B8ZS are enhancements of ........

  - a) Manchester

  - b) AMI

  - c) NRZ-I

  - d) RZ
Explanation: They modify AMI to prevent long zero sequences.

- Return-to-Zero (RZ) uses ........

  - a) Full-bit pulses

  - b) Half-bit pulses (returns to zero mid-bit)

  - c) No pulses

  - d) Triple voltage levels
Explanation: RZ pulses last half the bit duration, returning to zero for synchronization.

- NRZ-I encodes '1' as  ........

  - a) Fixed voltage

  - b) Voltage inversion from the previous level

  - c) Zero voltage

  - d) Positive voltage
*Explanation: NRZ-I = "Invert on 1". A '1' flips the signal; '0' keeps it unchanged.*

- Which scheme has no DC component?

  - a) Unipolar NRZ

  - b) Manchester

  - c) NRZ-L

  - d) All of the above
Explanation: Manchester’s symmetrical transitions average to zero DC.

- For the bitstream '1100', which line code has the most transitions?

  - a) NRZ-L

  - b) AMI

  - c) Manchester

  - d) Unipolar NRZ
Explanation: Manchester has a transition every bit (mid-bit), while others may not.

SECTION 3: CHARACTERISTICS & TRADE-OFFS

- Which scheme requires the least bandwidth?

  - a) RZ

  - b) Manchester

  - c) NRZ-L

  - d) AMI
*Explanation: NRZ-L uses 1 pulse/bit, while RZ/Manchester need 2 pulses/bit (double bandwidth).*

- Long sequences of zeros cause synchronization issues in  ........

  - a) Manchester

  - b) AMI

  - c) RZ

  - d) Differential Manchester
Explanation: AMI sends 0V for '0', so long zeros lack transitions for synchronization.

- Error detection is possible in  ........

  - a) NRZ-L

  - b) Bipolar AMI

  - c) Unipolar NRZ

  - d) All schemes
Explanation: AMI detects errors if two consecutive '1's have the same polarity (violation).

- True or False: Manchester encoding is used in classic Ethernet (10BASE-T).

  - True
*Explanation: 10 Mbps Ethernet uses Manchester for self-clocking.*

- Which scheme is most resilient to noise?

  - a) Unipolar NRZ

  - b) Differential Manchester

  - c) NRZ-I

  - d) RZ
Explanation: Differential schemes ignore absolute voltage levels, relying on transitions for noise immunity.

SECTION 4: ANALYSIS & COMPARISON

- Encode '1010' using NRZ-L:

  - a) +V, –V, +V, –V

  - b) +V, –V, +V, –V

  - c) 0V, +V, 0V, +V

  - d) –V, +V, –V, +V
*Explanation: NRZ-L: '1' = +V, '0' = –V (or vice versa).*

- Encode '110' using Manchester:

  - a) H→L, H→L, L→H

  - b) H→L, H→L, L→H

  - c) L→H, L→H, H→L

  - d) H→L, L→H, H→L
*Explanation: '1' = H→L, '0' = L→H. Always mid-bit transitions.*

- In AMI, the sequence '101010' is encoded as:

  - a) +V, 0V, –V, 0V, +V, 0V

  - b) +V, 0V, –V, 0V, +V, 0V

  - c) +V, –V, +V, –V, +V, –V

  - d) 0V, +V, 0V, –V, 0V, +V
Explanation: AMI alternates polarity for '1's and uses 0V for '0's.

- Which code guarantees a transition every bit?

  - a) NRZ-I

  - b) Manchester

  - c) AMI

  - d) Unipolar NRZ
Explanation: Manchester always transitions mid-bit for clocking.

- A scheme with 4 voltage levels can transmit:

  - a) 1 bit/symbol

  - b) 2 bits/symbol

  - c) 3 bits/symbol

  - d) 4 bits/symbol
*Explanation: 4 levels = log₂(4) = 2 bits/symbol.*

SECTION 5: ADVANCED CONCEPTS

- The "bipolar violation" in AMI indicates  ........

  - a) A zero sequence

  - b) An error

  - c) End of transmission

  - d) High bandwidth
Explanation: Two consecutive '1's with the same polarity break AMI rules, signaling errors.

- Which code replaces long zeros with special pulses?

  - a) NRZ-I

  - b) HDB3

  - c) Manchester

  - d) RZ
*Explanation: HDB3 (High-Density Bipolar 3) inserts pulses to break >3 zeros.*

- For optical fiber, which line code is preferred?

  - a) Manchester

  - b) NRZ

  - c) AMI

  - d) RZ
*Explanation: NRZ’s simplicity and low bandwidth suit high-speed fiber links (e.g., 100G Ethernet).*

- True or False: RZ has better synchronization than NRZ but uses more bandwidth.

  - True
Explanation: RZ’s mid-bit zero return aids synchronization but requires twice NRZ’s bandwidth.

- Which factor does NOT affect line code choice?

  - a) Bandwidth

  - b) Error detection

  - c) Colour of the cable

  - d) Synchronization
Explanation: Cable colour is irrelevant! Bandwidth, synchronization, and error resilience drive the choice.